# NETWORK DOCUMENTATION

Cisco Packet Tracer Lab: Connecting ACCOUNTS and DELIVERY Departments
Course: CCNA / Networking Fundamentals
Student Name:  Anup kumar singh 
Date: 01-12-2025

# 1. Objective

Design a network in Cisco Packet Tracer that connects the ACCOUNTS and DELIVERY departments.
The network must use the given network address 192.168.40.0 and ensure that:

✔ Each department has at least 2 PCs
✔ Proper number of switches and a router is used
✔ All devices are assigned correct IP address, subnet mask, and gateway
✔ Correct cabling
✔ Full connectivity between both departments (Ping must work)

# 2. Network Requirements (Based on Assignment)

a. Each department contains at least 2 PCs
b. Appropriate number of switches and one router must be used
c. Use network 192.168.40.0, configure IP, Subnet Mask, Gateway
d. Use correct cables (Straight-through)
e. Delivery PCs must ping Accounts PCs successfully

# 3. Subnetting

Given Network: 192.168.40.0/24

Since we need 2 departments → split into 2 subnets:

### Subnet 1 – ACCOUNTS

- Network: 192.168.40.0/25

- Range: 192.168.40.1 – 192.168.40.126

- Gateway: 192.168.40.1

- Subnet Mask: 255.255.255.128

### Subnet 2 – DELIVERY

- Network: 192.168.40.128/25

- Range: 192.168.40.129 – 192.168.40.254

- Gateway: 192.168.40.129

- Subnet Mask: 255.255.255.128

✔ Subnetting is correct
✔ Each department gets a separate /25 network

# 4. Network Topology Diagram

# 5. IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Router0 | G0/0 | 192.168.40.1 | 255.255.255.128 | - |
| Router0 | G0/1 | 192.168.40.129 | 255.255.255.128 | - |
| ACC PC0 | NIC | 192.168.40.3 | 255.255.255.128 | 192.168.40.1 |
| ACC PC1 | NIC | 192.168.40.2 | 255.255.255.128 | 192.168.40.1 |
| DEL PC2 | NIC | 192.168.40.130 | 255.255.255.128 | 192.168.40.129 |
| DEL PC3 | NIC | 192.168.40.132 | 255.255.255.128 | 192.168.40.129 |

# 6. Device Configuration

## Router Configuration

Router> enable

Router# configure terminal

Router(config)# hostname Router0

Router0(config)# interface gigabitEthernet0/0

Router0(config-if)# ip address 192.168.40.1 255.255.255.128

Router0(config-if)# no shutdown

Router0(config-if)# interface gigabitEthernet0/1

Router0(config-if)# ip address 192.168.40.129 255.255.255.128

Router0(config-if)# no shutdown

Router0# write memory

## Switch Configuration (Basic)

Switch> enable

Switch# configure terminal

Switch(config)# hostname Switch-ACCOUNTS

Switch-ACCOUNTS(config)# exit

Switch-ACCOUNTS# write memory

Set ip -> subnet mask -> gateway

(All values taken from ip addressing table )

# 7. Cabling Summary

| Connection | Cable Type |
| --- | --- |
| PC → Switch | Straight-through |
| Switch → Router | Straight-through |
| Switch ↔ Switch (if used) | Straight-through |

(Packet Tracer supports auto-MDI/MDIX)

# 8. Connectivity Test (Ping Results)

| From | To | Result |
| --- | --- | --- |
| PC2 (Delivery) → PC0 (Accounts) | ✔ Success |  |
| PC3 (Delivery) → PC1 (Accounts) | ✔ Success |  |
| PC0 → PC2 | ✔ Success |  |
| PC1 → PC3 | ✔ Success |  |

✔ All ping tests successful
✔ Both departments are fully connected

9. Conclusion

The network was successfully designed according to the assignment requirements:

✔ Proper subnetting using 192.168.40.0
✔ Correct IP addressing and gateways

✔ Correct router and switch configuration
✔ Correct cabling
✔ Full inter-department connectivity verified with ping